## 3D Model Print Instructions

Rocky Action Figure Instructions

This model includes 2 figure style option:

- Articulated Action Figure (1 body and 11 limb pieces)

- Statue (1 static posed figure)

Both model styles include two file options:

- Supported STL – Pre-supported and ready to print

- Unsupported STL – Ideal if you want to scale the model or create your own custom supports

The figure can be printed using either a resin or filament (FDM) 3D printer.
At 100% scale, the figure prints at approximately 3 inches tall.

## Printing Steps

### Step 1 – Choose your figure style option - statue or articulated figure

### Step 2 – Choose your file type

  - Option A: Supported STL

    - Supports are already included and optimized for printing

    - No additional edits are required before slicing

  - Option B: Unsupported STL

    - Scale the model to any size you prefer

    - You will need to generate supports before printing

    - Most slicers offer both automatic and manual support options

- Step 3 – Import file
Download and import the STL file into your slicer program (such as Chitubox, Bambu Studio, or Lychee).

- Step 4 – Prepare the print
Use your slicer to generate the print layout and settings required for your specific 3D printer.

- Step 5 – Post-Print
Once printing is complete:

- Carefully remove the supports

- For easier removal, place the model in warm water to soften the supports and reduce surface damage

Pro Tips:

- For all prints-

  - Resin prints will capture finer detail than filament prints

  - For best results, print at a 0.03–0.05 layer height (resin) or 0.1–0.2 (FDM)

  - Always clean resin prints in isopropyl alcohol or similar resin cleaning product fully after printing.

  - If printing in resin make sure to cure the figure with UV light based on your resins recommendations.

- Articulated figure print-

  - For optimal resin printing you will want a resin with a mix of strength and elasticity. A recommended mixture for this figure with moving parts is ½  Resione Th-72 mixed with ½ Tough74.  Resins that don't have enough elasticity may be too brittle for this figure.

  - When assembling the articulated version you may want to heat up the cavities with warm water or a blow dryer for easier assembly.

  - See the labeled diagram below for proper assembly of the articulated figure.

  - There are 2 options for his left hand that you can choose from.  An open hand (1C)  or closed hand (1B).